MACH3 USB 4 axis control board

Control card features:

支持Mach3 所有版本。

Supports all versions of Mach3.

支持所有Windows版本，包括目前最新版Windows7, Windows8。

Supports all versions of Windows, including the latest version of the current Windows7, Windows8.

支持4轴联动，包括点动4轴联动。

In support of the 4 axis linkage, including 4 axis linkage.

支持自动对刀，电子手轮，软件限位，软件消回差功能。

Support for automatic tool setting, electronic handwheel, software limit, software return difference eliminating function.

拥有50KHz输出，接伺服/步进。

Has 50KHz output, the servo / step.

拥有16个输入16个输出端口。

With 16 input 16 output ports.

1, the plug-in is copied to the MACH3 installation of the software in the PlugIns folder

1, connection control board for USB line, the computer promptsfound hardware, install the driver

A, you select the driver, B, install from disk. C, browse the driver file location. D, find the file "kitstup". E, in the device manager can see this card is installed。

1, software settings and open software will have the following tips is to determine, or call "don't ask"

1, connection control card, if the software emergency stop buttonflashing, to set the input ESTOP

"Enable" and "low level" some other effective up, can not be provided, if the external button stop back again

5．The stepping motor is provided.

"Enable" up "direction, low effective" and "pulse low effective"according to your stepping motor driver selection, recommend other items can not set up.

6，Pulse millimeter settings according to your ownsituation

7. Output settings, a relay board, the 2 core socket is relay point, can do to control spindle。

Example: Set: "enable" remind, pin is#1, active low, other settings

A #2--#15 output end plate, can all be output, active low.

8, the input set can be limiting, external OEM button, from #17---#24with optocoupler connects the socket, #25---#31 can also be input.

Patients with A, set the OEM do. Stop: OEM#1, #23input control card。

1021 is the stop code

Short circuit negative and #23 can make the external emergency stop button to do

9, do limit setting can refer to the following settings

10 stepper motor wiring: negative generally not connection, or with aswitch to control the active low enable terminal

11 reset buttons and indicator

Another button and indicator lights temporarily not used.

。

12, CP and PWM is a charge pump and spindle speed control terminal, now can not use

13, the extended USB regulated power supply module card, the card not external power supply is to use the notebook can also work. The extended optical card, if the expansion card power coupler and not completely isolated, if use the external power supply, the jumper can select the input and output true completely isolated 。