- nstallation

2、 Debugging

1.  Inatall CAMERA SENSOR,Position and direction should be as the shown below. 

相机旁边的俯仰臂上 Pitch beside the camera

2安装好FRAME SENSOR,方向，位置也必须和下面的图一样 Install FRAME SENSOR, position and direction should be as shown below

Rear of the rolling motor

Note: The position and direction of the two sensors should be as the picture shown, otherwise, you have to modify the direction of the two SENSORs yourself that in the GUI. If the control board you using is produced by other factory, please confirm the direction of the SENSOR carefully then.

3. Install the control board

Make sure the motor is placed correctly. 1. The motor beside the camera should be connected to the control board PITCH, the motor behind the camera to control board ROLL; the motor above the cam to control board YAW.

- Download GUI: http://www.basecamelectronics.com/downloads/, and follow the prompts to install JAVA.

- Install USB driver and check the attachment.

- Debugging the parameter

- After supplying power to the control board,conncet it to computer with USB.

6.1 Choose the right COMx,make connection then.

6.2 Click LOAD, loading Thanksbuyer-20151123.profile, and click WRITE

6.3 Click IMU Calibration Helper , calibrate the two SENSORs,

Accelerometer and Gyro needs RESET and CALBRATE, after calibrating, CLOSE

6.4 Click AUTO, and let the system search the correct motor direction automatically.

Note:If the stabilizer is imbalance, calibrations are necessary.

6.5.  For different modes, only needs to modify the settings in the FOLLOW to achieve the full following mode.

Pitch locking mode

Course locking  mode