F218

Standalone Biometric time attendance INSTRUCTION MANUAL

CONTENT

Introduction	4

Supplied Part List	4

About	4

Hardware Specification	4

Precaution	5

Product Overview	6

F218 Illustration:	6

Product Overview	6

User Management	7

User Enrollment	7

Illustration of Correct Fingerprint Pressing	7

Wrong Pressing Method	8

Authentication & verification	8

Authentication with Fingerprint	8

Authentication with RF ID card	8

Authentication with Password	8

Delete User	9

Menu Level Structures	9

Menu 1 User / Manager	9

Menu 2 Delete User	11

Menu 3 Data Download	11

Menu 4 Time Setting	12

Menu 5 Advance Setting	12

Device Setting	13

Record Setting	14

Door Lock Out	15

Menu 6 Information	16

Device Operation for Administrator	18

Device Management	18

New User Registration / New Administrator Registration	18

Enrolled User Registration (Backup Register)	20

Password Registration	20

Card Registration	21

Delete User	22

Attendance Log and System Log Management	23

Query of Attendance Log	23

Query of System Log	24

Delete all Record	25

Advance Setting	25

Device Setting	26

Admin Quantity	26

Language	27

Voice Out	27

Volume	28

Auto Power Off	28

Screen Saver	28

RS485 Download	29

Verify	29

Default Setting	29

Record Management	30

Administrator Record (System Log) Warning	30

User Record (Attendance Log ) Warning	31

Reverify	31

Record Detail	31

Enroll Information (Storage Detail)	32

Maintenance Clean	33

Clean keyboard & LCD Screen	33

Clean Optic Lens	33

Quick Setup Guide	33

Door hardware connection diagram:	34

# Introduction

## Supplied Part List

- Fingerprint Terminal	1 pc

- Installation Screw Bag	1 bag

- Mounting Panel	1 pc

## About

F218 biometric time attendance terminal is a flexible and reliable design to meet the market requirement for a secure and cost-effective biometric time attendance solution. It allows registering maximal 3000 fingerprints, 3000 passwords, 3000 RF ID cards (optional) and saving maximal 80,000 records.

## Hardware Specification

| Fingerprint Template Capacity | 3000 |
| --- | --- |
| Password (PIN) Capacity | 3000 |
| RF ID Card Capacity | 3000 (optional) |
| Record Capacity | 80,000 |
| Verification Mode | 1:1 or 1:N |
| Communication | TCP/IP, RS485 & U-disk Via U-disk it can send fingerprint data to other F218 |
| Recognition Mode | Fingerprint or password or RF ID Card (optional) |
| Keyboard and Display | 2.8 inch color screen and Touch button |
| Power Supply | 12VDC; standby current: 0.25A; operating current: 0.3A when not running electric strike lock |
| Identification speed | ＜=0.7 second |
| FRR (False Rejection Ratio) | ＜=0.01% |
| FAR (False Acceptance Ratio) | ＜=0.0001% |
| Operating Environment | Temperature: 0°C – 40°C Humidity: 20% – 80% |
| Menu Language Support | In both Chinese and English and other languages |
| Dimension | 219(L) x 82(W) x 27(D) mm |

## Precaution

Don’t install the device under direct strong sunlight. Strong sunlight affects collecting of fingerprint and it may cause failure on the fingerprint authentication.

During summer it’s highly recommended not to use outdoors. The device working temperature scope is 0-40 0C. Long time using outdoors plus the heat of device itself may lead to slow reaction of device and less pass rate. Once it’s necessary to use outdoors, a sunshade and a set of cooler radiation are recommended.

As the fingerprint identification will be affected by the weather, dry climate will not facilitate the fingerprint identification. So in dry weather conditions, it will be recommended to blow at the finger to moisten the fingerprint, which is helpful to  enhance and ensure the identification result in dry climate.

# Product Overview

## F218 Illustration:

## Product Overview

Communication

F218 has U-disk, RS485 and TCP/IP communication interfaces .

Keypad Functions

ESC			Move one level up from the menu hierarchy

M/OK		Get into the system setting menu / Confirm the current setting

▲	         Move the cursor up

▼			Move the cursor down

◄           Move the cursor  to the left

►             Move the cursor to the right

0...9			Numeric keys

## User Management

Two user roles are defined: namely manager or normal user .

- Normal users can clock on the device but can NOT access the settings.

- Managers can clock on the device as well as run system management :

1. Register/delete normal users' fingerprints, passwords and RF ID cards (optional).

2. Browse attendance records as well as other system information.

3. Set up system configurations

## User Enrollment

Enrollment is the process of creating an ID, assigning a password and scanning a user’s fingers to create the fingerprint templates. All these are stored in the device.

During user verification, the stored fingerprint templates are compared to the current scanned fingerprint to confirm the user’s identity.

Enrollment takes approximately 2 seconds for each user. Up to 9 fingers can be enrolled for the same ID number. Ideally, at least one finger on each hand should be enrolled in case of injures.

Illustration of Correct Fingerprint Pressing

Flatly press fingerprint on U fingerprint scanner

Try to press fingerprint center to the middle or a little upper of U scanner

Wrong Pressing Method

Vertical

Too partial

Slant

Too lower

Note: Please adopt correct fingerprint placement method, to ensure the best identification results.

## Authentication & verification

There are 3 verification ways to access the unit, either by Fingerprint or by Password or by RF ID card (optional).

### Authentication with Fingerprint

Press the finger directly on the scanner and wait until the registered ID number is displayed on the LCD screen. If unsuccessful, press the finger accurately one more time.

### Authentication with RF ID card

Swipe the RF ID card to the device (place it over fingerprint sensor position)

### Authentication with Password

Enter the enrolled ID No. at the device and press OK to confirm.

“1:1 check P” will display on the screen and then input the correct password and wait until the registered ID number displays on the LCD screen.

If unsuccessful, “Wrong Password” will display and it’s required to input correct password once again.

## Delete User

Press menu and select Delete User, press OK and input the enrolled ID number which is to be deleted and then press OK to confirm, then the relevant user is deleted.

# Menu Level Structures

The first level menu items show up when MENU key is pressed if there’s no available administrators, administrator affirmation will be done otherwise.

Menu 1:Register User

The Item “Register User” includes 4 Sub Items which are “User”, “Admin”,

“Security level” and “U-disk Upload”

The Sub Items will show up after user select “Register User”, which under the first level menu, and Press “OK”

REGISTER USER

1 USER

3 Security Level

2 Admin

4 U-disk UPLOAD

## Menu 1 User / Manager

Select the level of User registration (User/Manager) and Press “OK”. The Third Level Menu Items show up as the figure below. “New Register” is for new user registration. “Backup Register” is for enrolling “Backup Record” of registered user.(PS: Back Record means the 2nd or 3rd Finger Print Template and password) The registration method shows up after “New Register” or “Backup Register” is selected.

Registration of administrator, first needs to register the Super Administrator, then you can register the USB administrator, Enroll Administrator and General Administrator.

Administrator

USB Administrator

Enroll Administrator

General Administrator

Super Administrator

New Register RRTTENJKE LRERRRRegister

Backup

Note: Registration card options are optional, with card machines have card options.

USER

New Register

Backup

Remark

Register abbreviated to REG

Fingerprints abbreviated to FP

Password abbreviated to PASS

All card operation in this menu is not applicable to F218

There are 3 options under Security Level which are 1:1 Verify, Init.1:1,1:N Identify

1:1 Verify - Compare the user’s fingerprint and the fingerprint template stored in card.(Option Setting depends on the model)

1:N Identify- Compare the user’s fingerprint with the fingerprint template of all registered users stored in machine.

U-disk Upload

The Backup of Menu Setting can be uploaded from U-disk storage device, namely U-disk flash disc to the Fingerprint Machine.

## Menu 2 Delete User

The Delete Menu shows up after selecting Delete User under the First Level Menu and pressed OK. Enter the user ID or arrow keys to select the user ID, Press OK to confirm the deletion.

Delete

ID.         00000001

ESC-ESC       SET-OK

## Menu 3 Data Download

The Sub Items show up after select Data Download of user First Level Menu

DATA Download

1 U-disk Download

2 RS232

3 Ethernet Download

Remark:

Download abbreviate to dwnld.

- U-disk download

“There are five options under the U-disk Download, which are “Glog” and “All Glog” and “Slog” and “All Slog” and “All Enroll Data”

Glog     - Download the new record

All Glog  - Download all the records

Slog     - Download the new management record

All Slog  - Download all the records management

All Enroll Data  - Download all the registered data

2) Ethernet Dwnld

6 Server IP

7 SerPrtNo      7005

5 Server Req     YES

4 Port No        5005

3 Default gateway

2 Subnet mask

1 IP Address

Ethernet Dwnld

## Menu 4 Time Setting

The Time Setting Menu shows up after select Time Setting under the First Level Menu and pressed OK

Current Time

2009-9-12

14:22:25

WED

Move the cursor location by direction key. Enter the correct value by number key. (Week is calculated by the software, without input.)

## Menu 5 Advance Setting

There are three items under Advance Setting, which are Machine Setting, Record Setting and Door lock out.

1 Device Setting

Device Setting configures the setting of machine operation.

Record Setting configures the setting of data logging.

Door Lock Out is the setting for Access Control unlock time and purview to the users

already enrolled.

5 Volume          8

10 Default Setting

9 Verify         F/P/C

8 RS485 Dwnld   Yes

7 Screen Saver   NO

6 Auto Off        NO

4 Voice Out      Yes

3 Language    English

2 Admin Qty       5

1 Machine ID      1

Device Setting

Device Setting

Machine ID.: The Unique ID of the machine on the same communication channel.

Admin Qty.: The maximum number of machine administrator.

Language:  The language of operation menu. It supports multiple languages.

Voice Out:  Enable or disable the function of voice indication.

Volume:  Set the prompt voice volume.

Auto Off:  The machine will shut down after a certain idle time.

Screen Saver: For the protection of the screen

RS485 Dwnld: Enables or disables RS485 Download

Verify:  Choose verification mode.

Default Setting: Restore all setting which under Settings to factory default setting.

### Record Setting

Arec wrn(Slog Wrn): To set the limit of remaining system log memory. If the number of remaining system log memory reach the limit. Warning occurs.(All administrators’ operation is record as system log.)

Urec wrn(Glog Wrn): To set the limit of remaining attendance record memory. If the number of remaining record memory reaches the limit. Warning occurs.(All attendance activity is record as Glog.)

Reverify: The duration of 2 successful attendances which from the same fingerprint . All duplicate attendance record doesn’t be recorded.

### Door Lock Out

Door Lock Out setting are for setting the access control. All the setting can be set from the ACCESS CONTROL module of software and then uploaded into the device, or can be directly set on the individual device.

Time Setting:  Time setting from hardware, which is only for one separate device, and it               can also be set from software access control part and then uploaded to 1or multiple devices.

Lock Register:  Set lock register of different ID number. It can also be set from software.

Time Group:   Time Group setting from hardware, which is only for one separate device, and it can also be set from software access control part and then upload to 1 or multiple devices.

Unlock Group:   Unlock setting of time group. It can also be set from software.

Unlock Time:   Unlock time delay and it counts by second. It can also be set from software.

Lock Type:   A.  Alarm type: NG for none, No for Normal Open, NC for Normal Close.

B.  Alarm Delay: Alarm time delay and it counts by seconds.

Access Mode:  It includes G Mode & Users. G Mode is defaulted as NO, if it’s changed as Yes, and Users qty is 2, it means it need any 2 person who have already registered on site to open the door together.

## Menu 6 Information

Information shows the storage detail, records, manufacturer and firmware version.

The sub items are shown below.

Storage Detail:  To show the information about of user registration such as the    number of registered users, number of registered administrators, number of fingerprint templates, number of registered passwords and number of registered Cards.

Record Detail:   To show or operate the Device record such as number of user records, number of administrator records, attendance record of users, system log of administrators, and delete all record.

System Detail:  To show hardware information which includes Manufacturer, Device Name, Release Time, Serial Number, Firmware version, Software Version and Upload firmware .

Information

1 Storage Detail

User Reg.

Manager Reg.

Finger

Password

Card

2 Record Detail

View User Rec.

View User Rec.

Delete All Rec.

Device Name

Firmware Version

Software Version

Upload Firmware

3 System Detail

Manufacturer

Release Time

Serial Number

# Device Operation for Administrator

How to show the First Level Menu

If the manager confirm is requested on the operation and the user didn’t enroll any administrator, please shut down the Device and contact with retailer.

If confirmation is failed, Illegal is shown and request the reconfirmation.

## Device Management

### New User Registration / New Administrator Registration

Step1  Enter the First Level Menu(Refer to Page 6.)

Step2  Press 1 to choose Register User

Menu

1 register User

2 Delete User

3 Data Download

4 Time Setting

5 Advance Setting

6 Information

Step3  Press 1 to choose User for normal user registration Or Press 2 to choose  Administrator registration(The administrator menu has four different levels, first need to register the super administrator.)

Level Selection

1 User

2 Administrator

3 Security Level

4 U-disk Upload

Administrator

1 USB Administrator

2 Enroll Administrator

3 General Administrator

4 Super Administrator

Step 4  Press 1 to choose New Register

Enroll Method

1 New Register

2 Backup Register

Step 5  Press 1 to choose Reg Finger

Enroll Object

1 Reg Finger

2 Reg Password

3 Reg Card

Step 6  Enter the User ID(i.e. )00000001 and Press OK

Reg Finger

ID  00000001

ESC-ESC    SET-OK

Step 7  Flatly press fingerprint the first time on U fingerprint collector

Press Finger

First

Step 8 Flatly press fingerprint the second time on U fingerprint collector

Press Finger

Second

Step 9 Flatly press fingerprint the third time on U fingerprint collector

Press Finger

Third

Successful Registration

Step 10 Press ESC to quit the registration process.

Reg Finger

Continue?

NO-ESC      YES-OK

### Enrolled User Registration (Backup Register)

It allows administrator enroll 2 more fingerprint temple, password and Card for the same user as Backup.

### Password Registration

Step 1 Enter the Menu Enroll Object

Step 2 Press 2 to choose Reg Password

Enroll Object

1 Reg Finger

2 Reg Password

3 Reg Card

Step 3 Enter the User ID (i.e) 00000001 and Press OK

Reg Password

ID    00000001

ESC-ESC      SET-OK

Step 4 Enter the Password (1-8 digits) and Press Enter.

Reg Password

ID      00000001

PWD In  XXXX

ESC-ESC    SET-OK

Step 5 Re-enter the password to confirm

Reg Password

ID     00000001

PWD Re Input   XXXX

ESC-ESC     SET-OK

Successful Registration

Step 6 Press ESC to quit the registration process.

Reg Password

Continue?

NO-ESC      YES-OK

### Card Registration

Step 1 Enter the Menu Enroll Object

Enroll Object

1 Reg Finger

2 Reg Password

3 Reg Card

4 Reg FP & Pass

5 Reg FP & Card

Step 2 Press 3 to choose Reg Card

Reg Card

ID      00000001

ESC-ESC    SET-OK

Step 3 Enter the User ID (i.e) 00000001 and Press OK

In Card No.

Swipe Card!

ESC-ESC    SET-OK

Step 4 Please swipe card on the detecting area.

Step 5 Card ID is shown on the screen and Press OK to confirm.

In Card NO.

50741209

ESC-ESC   SET-OK

Successful Card Registration

Step 6 Press ESC to quit the registration process.

## Delete User

Step 1 Enter the First Level Menu( Refer to Page 6)

Step 2 Press 2 to choose Delete User

Menu

1 Register User

2 Delete User

3 Data Download

4 Time Setting

5 Advance Setting

6 Information

Step 3 Enter the User ID and Press OK

Delete

ID      00000001

ESC-ESC    SET-OK

Step 4 Press OK to confirm

Delete

00000001

Sure to Delete?

ESC-ESC     SET-OK

Successful Deletion

If any backup record belongs to the deleted User ID. the alert of deletion of backup record will show on the screen.

Delete

00000001-1

Sure Delete?

ESC-ESC      SET-OK

The type of record is indicated by the last digit

0 means 1st Fingerprint template

1 means 2nd Fingerprint template

2 means 3rd Fingerprint template

P means Password

C means Card

## Attendance Log and System Log Management

### Query of Attendance Log

Step 1 Enter the First Level Menu (Refer to Page 6)

Step 2 Press 6 to choose Information

Menu

1 Register User

2 Delete User

3 Data Download

4 Time Setting

5 Advance Setting

6 Information

Step 3 Press 2 to choose Record Detail

Information

1 Storage Detail

2 Record Detail

3 System Detail

Step 4 Press 1 to choose View User Rec

Record Detail

1 View User Rec

2 View Admin Rec

3 Delete All Rec

Step 5 Press OK to query all attendance log(In order by time) Or Enter the User ID to query a certain user’s record. Press OK

View User Rec

ID    00000001

ESC-ESC  Continue-OK

Step 6 Press ▲ or ▼ to view the previous or next record

View User Rec

N/C

09/09/26    13:23

00000001

ESC- ESC    Con-OK

C:         total amount of attendance record (C-00022 means there are 22 records)

N:         the sequence of attendance record(n-00001 means it is the first record)

00000001:      User ID of the current record

09/09/26 13:23 :  attendance time

### Query of System Log

Step 1 Enter the Menu Record Detail

Step 2 Press 2 to choose View Admin Rec

Record Detail

1 View User Rec

2 View Admin Rec

3 Delete All Rec

Step 3 Press OK to query all system log.(In order by time)or enter the User ID to query a certain user’s system log. Press OK

View Admin Rec

ID    00000001

ESC-ESC  Continue-OK

Step 4 Press ▲ or ▼ to view the previous or next record.

View Admin Rec

N/C

09/09/26    13:23

ESC- ESC    Con-OK

C:         total amount of attendance record (C-00022 means there are 22 records)

N:         the sequence of attendance record(n-00001 means it is the first record)

### Delete all Record

Step 1  Enter the Menu Record Detail

Step 2  Press 3 to choose Delete All Rec

Record Detail

1 View User Rec

2 View Admin Rec

3 Delete All Rec

Step 3  Press OK to confirm the deletion

Delete All Log

Sure to delete?

NO-ESC     YES-OK

Successful Deletion

## Advance Setting

Follow the steps below to access Device Setting Menu.

Step 1  Enter First Level Menu

Step 2  Press 5 to choose Advance Setting

Menu

1 Register User

2 Delete User

3 Data Download

4 Time Setting

5 Advance Setting

6 Information

Step 3  Press 1 to choose Device Setting

Advance Setting

1 Device Setting

2 Record Setting

3 Door Lock Out

### Device Setting

The Machine ID of each Device on the same channel should be unique (No matter Using RS485 or TCP/IP protocol)

Step 1 Press 1 to choose Device Setting

Device Setting

1 Machine ID     1

2 Admin Qty      5

3 Language     English

4 Voice Out      Yes

5 Volume         8

Device Setting

6 Auto off         No

7 Screen Saver     No

8 RS485 Dwnld    Yes

9 Verify          F/P/C

10 Default Setting

Step 2 Enter the NEW Number with range between 1-255(i.e. 2) and press OK to confirm.

Device Setting

1 Machine ID     1

2 Admin Qty      5

3 Language     English

4 Voice Out      Yes

5 Volume         8

### Admin Quantity

Step 1  Press 2 to choose Admin Qty

Device Setting

1 Machine ID     1

2 Admin Qty      5

3 Language     English

4 Voice Out      Yes

5 Volume         8

Step 2  Enter the maximum no of Device administrator(i.e. 6) and press OK to confirm

Device Setting

1 Machine ID     1

2 Admin Qty      5

3 Language     English

4 Voice Out      Yes

5 Volume         8

### Language

Step 1  Press 3 to choose Language

Device Setting

1 Machine ID     1

2 Admin Qty      5

3 Language     English

4 Voice Out      Yes

5 Volume         8

Step 2  Press ▲ or ▼ to choose Language (English, Espanol, Simplified Chinese and Traditional Chinese) and press OK to confirm.

Device Setting

1 Machine ID     1

2 Admin Qty      5

3 Language     English

### Voice Out

Step 1  Press 4 to choose Voice Out

Device Setting

1 Machine ID     1

2 Admin Qty      5

3 Language     English

4 Voice Out      Yes

5 Volume         8

Step 2  Press ▲ or ▼ to enable or disable the function

Yes  - Enable

No   - Disable

Press OK to confirm

Device Setting

4 Voice Out      No

5 Volume         8

6 Auto Off        No

### Volume

Step 1  Press 5 to choose Volume

Device Setting

1 Machine ID     1

2 Admin Qty      5

3 Language     English

4 Voice Out      Yes

5 Volume         8

Step 2  Enter and press OK to confirm (1- Minimum ,8- Maximum)

### Auto Power Off

Step 1  Press 6 to choose Auto Off

Device Setting

6 Auto off         No

7 Screen Saver     No

8 RS485 Dwnld    Yes

9 Verify          F/P/C

10 Default Setting

Step 2  Press ▲ or ▼ to disable the function or set the delay time (1--- 255mins)

### Screen Saver

Step1  Press 7 to choose Screen Saver

Device Setting

6 Auto off         No

7 Screen Saver     No

8 RS485 Dwnld    Yes

9 Verify          F/P/C

10 Default Setting

Step 2  Press ▲ or ▼ to disable the function or set the delay time (1---- 255s)

### RS485 Download

Step1  Press 8 to choose RS485 Dwnld

Device Setting

6 Auto off         No

7 Screen Saver     No

8 RS485 Dwnld    Yes

9 Verify          F/P/C

10 Default Setting

Step 2  Press ▲ or ▼ to enable or disable the function

Yes  - Enable

No   - Disable

Press OK to confirm

### Verify

Step1  Press 9 to choose Verify

Device Setting

6 Auto off         No

7 Screen Saver     No

8 RS485 Dwnld    Yes

9 Verify          F/P/C

10 Default Setting

Step 2  Press ▲ or ▼ to chose verify mode.

### Default Setting

Step 1  Press 10 to choose Default Setting and confirm the restoring.

Device Setting

6 Auto off         No

7 Screen Saver     No

8 RS485 Dwnld    Yes

9 Verify          F/P/C

10 Default Setting

All setting under machine setting will be recovered to default factory setting (Except System Log and Attendance Log)

## Record Management

Follow the steps below to access Record Setting Menu.

Step 1  Enter First Level Menu

Step 2  Press 5 to choose Advance Setting

Menu

1 Register User

2 Delete User

3 Data Download

4 Time Setting

5 Advance Setting

6 Information

Step 3  Press 2 to choose Record Setting

Advance Setting

1 Machine Setting

2 Record Setting

3 Door Lock Out

### Administrator Record (System Log) Warning

Step 1  Press 1 to choose ARec Wrn

Record  Setting

1 ARec Wrn       100

2 URec Wrn       1000

3 Reverify         No

Step 2  Enter the number of residue system log memory(1-255) before the warning occur,and press OK to confirm.(i.e. 100 means the warning will occur if the residue system log memory reach 100)

### User Record (Attendance Log ) Warning

Step 1  Press 2 to choose URec Wrn

Record  Setting

1 ARec Wrn   100

2 URec Wrn   1000

3 Reverify     No

Step 2  Enter the number of residue attendance record(1-1500) before the warning occur. And press OK to confirm. (i.e. 1000 means the warning will occur if the residue attendance log memory reach 1000)

### Reverify

Step 1  Press 3 to choose Reverify

Record  Setting

1 ARec Wrn   100

2 URec Wrn   1000

3 Reverify     No

Step 2  Press ▲ or ▼ to disable the function or set the duration between 2 successful attendance (1-255mins)

## Record Detail

Follow the steps below to access Information Menu.

Step 1 Enter First Level Menu

Step 2 Press 6 to choose Information

Menu

1 Register User

2 Delete User

3 Data Download

4 Time Setting

5 Advance Setting

6 Information

Step 3 Press 2 to choose Record Detail

Information

1 Storage Detail

2 Record Detail

3 System Detail

Step 4 Press OK to view details

Record Detail

1 View User Rec

2 View Admin Rec

3 Delete All Rec

### Enroll Information (Storage Detail)

It shows all amount of Enrolled record storage such as the number of enrolled user,the number of enrolled administrator, enrolled fingerprint template, enrolled password and enrolled card.

Step 1 Press 1 to choose Storage Detail

Information

1 Storage Detail

2 Record Detail

3 System Detail

Step 2 Press 1 to choose User Reg

Storage Detail

1 User Reg      4

2 Manager Reg   3

3 Finger         1

4 Password      0

5 Card          0

# Maintenance Clean

Sometimes optical lens, keyboard or screen needs to be cleaned. Due to different outside working environment, it’s hard to give an exact cleaning schedule. The below table is a guide that may help:

| c |  |
| --- | --- |
| Keyboard & screen | Whenever it’s too dirty and blurry on the surface, it needs cleaning |
| Optic Lens | Don’t clean too frequently. Lens works better with a little lipid. Once the lens is shaded or somewhat affects identification of fingerprint, it needs cleaning. |

# Clean keyboard & LCD Screen

Make sure to turn off the device when it needs to do cleaning on keyboard & liquid screen, Use wet duster cloth or other mild detergent and then dry it.

# Clean Optic Lens

Follow below suggestion when cleaning Optic Lens;

- Once it has dust or grit, please blow dust off first

- Use adhesive tape to clean display window.  Warning: Don’t use water or other detergent to clean which may do harm to optic lens.

- Wipe with fine cloth without any fluff and be careful not to hurt lens. Once it has fluff on the lens, blow it off after the lens becomes dry.

# Quick Setup Guide

Hardware

Power on/off

- Connect the power Adapter to a 240V/110V socket.

- Connect the fingerprint terminal, and then it will automatically power on.

# Door hardware connection diagram: