ALL to SDI Scaler Converter

Operation Manual

- Introduction

The ALL to SDI Scaler Converter allows you to upscale Composite,VGA,DVI,HDMI signals to high definition video of SDI formats (HD-SDI SMPTE 292M or 3G-SDI 424M/425M), then output to both ports at the same time.

- Features

- Supports HDMI, DVI, VGA and Composite Video input signals;

- Supports two SDI output simultaneously;

- Supports SDI output distance up to 100 meters (3G-SDI), or 200 meters (HD-SDI);

- Scales any input signal to the HD-SDI or 3G-SDI output signals;

- Supports HDMI input timings: VGA@60 ~ SXGA@60 & 480i ~1080p@60 Hz;

- Supports VGA/DVI input timings: VGA@60 ~1920×1080@60 Hz;

- Supports Composite Video input timings NTSC, PAL,SECAM;

- Supports Black screen signal output;

- Package Contents

  - ALL to SDI Scaler Converter         1pcs

  - 12V/2.5A DC power adaptor        1pcs

  - Operation Manual                 1pcs

- Specification

SDI Transmission Rates    2.970 Gbps, 1.485 Gbps
Video Bandwidth          165 MHz
Input Ports               1×HDMI (Type A connector),

1×DVI with Audio

1x VGA with Audio
                         1×Composite with R/L
Output port              2×SDI (HD/3G-SDI), 
SDI Cable Distance        3G/HD/up to 100/200 meters with
                         BELDEN 1694A cable
HDMI Cable Distance       Input: 10 m, 8/12-bit
Input HDMI Timing Support  VGA@60, SVGA@60, XGA@60, SXGA@60,
                            WXGA@60,480i@60,576i@50, 480p@60,
                            576p@50, 720p@50/60, 1080i@50/60,
                            1080p@24/25/30/50/60;
Input VGA/DVI Timing Support  VGA@60, SVGA@60, XGA@60,
                               SXGA@60, WXGA@60,1280x720@60,

1920×1080@60
Input Composite Support    NTSC, PAL,SECAM
Output Timing Support     SDI:  SMPTE 292M (1.485 Gbit/s) &

SMPTE 424M/425M (2.970 Gbit/s)
ESD Protection Human-body Model:
                         ±8kV (air-gap discharge)
                         ±4kV (contact discharge)
Dimensions             110mm (W)×288mm (D)×32 mm (H)
Weight                  750 g
Chassis Material         Metal
Operating Temperature   0˚C~40˚C / 32˚F ~ 104˚F
Storage temperature     -20˚C~60˚C / -4˚F ~ 140˚F
Relative Humidity        20~90% RH (no condensation)
Power Consumption      5W

- Operation and Fuctions

5.1 Front panel

- INPUT BOTTONS: Press these buttons to select the input source.

- RESOLUTION BOTTONS: Press these buttons to select the SDI output resolution.

5.2 Rear panel

- OUTPUT: The two SDI output is where you connect the SDI monitor with SDI cable for input source display.

- Input: These slot is where you connect the HDMI/DVI/VGA/Composite source output from DVD, PS3,Set-top Box and Note Book.

- DC 12V: Plug the 12V DC power supply into the unit and connect the adaptor to AC wall outlet.

- ON/OFF: Power ON/OFF.

6. Connection Diagram