OBDII Break Out Box

User manual

Dear user,

Thank you for purchasing OBDII Break Out Box. This manual contains

introduction, function, usage and after-sale, please read it carefully before

using.

CONTENT

I. Introduction ............................................................................................. 01

1.1 Overview of Test Box ........................................................................ 01

II. Function ................................................................................................. 02

2.1 Features of Test Box ............................................................... 02

2.2 Indicator Light Meaning ..............................................................................

III. Usage.................................................................................................... 04

3.1 Sonata Relay Testing Demonstration ....................................................... 04

FAQ .................................................................................................................. 11

III. After-sale ................................................................................................ 11

- 1 -

I. Introduction

1.1 Overview of  Test  Box

OBDII Protocol Detector & Break Out Box is our latest R & D automotive fault

diagnostic tools. It’s mainly used for changing automotive diagnostic OBD

stitch to signal judgment level transfer. It can connect with any diagnostic

instruments and judge whether car communication is normal. Also it can

supply car power during replacing the battery , in order to prevent the body

electrical equipment locked automatically.

II. Function

2.1 Features of Test Box

1. Test and find out the connection fault of OBD2 interface when using the tool.

2. Monitor OBD interface voltage and ground wire so that it can display low

voltage and fault diagnose stitch.

3. Used for OBD interface signal jumping and quickly test if every diagnostic

stitch communication of OBD diagnostic interface is OK.

4. Save automotive data and prevent automotive data losing and electric

device locking.

- 2 -

2.2 Indicator Light Meaning

1. Manufactory custom

2. SJE1850 wire<bus+>

3. Manufactory custom

4. Power grounding

5. Signal grounding

6. SAEJ2284《CAN》high

7. ISO9141-2&iso, DIS4230-4K wire

8. Manufactory custom

9. Manufactory custom

10. SJE1850 wire《bus->

11. Manufactory custom

12. Manufactory custom

13. Manufactory custom

14. SAEJ2284《CAN》low

15. ISO9141-2 & ISO/DIS 4230-4L wire

- 3 -

16. Car battery positive

III. Usage

3.1 Actually testing instructions

1. In series connect OBD signal adaptor

and car fault diagnostic tester. It supples

power normally if its storage battery

voltage LED displays more than 10V

and NO.16, NO4 or NO.5 LED light.

It’s shortage of storage battery voltage if

less than 10V. None of LED turning light

means that there is something wrong

with the diagnostic holder power

suppling, then check whether abnormal

with the holder and storage battery

voltage.

2. The communication signal LED will

turn light during testing (Egg: NO. 7 wire

signal). The testing of NO.7 pin is

normal when LED flash,it means

communicate normally.

3.The car diagnostic communication is

NO. 15 wire but diagnostic tool is NO. 7

wire, it can’t test car in normal. OBD

adaptor can connect NO.7 wire and

NO.15 wire. When it test, NO.7 and

NO.15 LED will turn light and flash. It can

normally test.

3.2 Replay car storage battery

1. Connect the OBD signal adaptor and the car diagnostic holder, NO. 4 and

NO.6 LED light,with 12V.

2. Supplying power to NO.4 negative pin and NO.6 positive pin. Then the OBD

adaptor can supply power to the car through diagnostic holder positive and

negative, to prevent car data losing and audio locking.

3. Remove the car battery.

4. Swift off the power and OBD adaptor in order to finish battery changing.

FAQ and solutions:

1. Make sure the vehicle is parking and has electricity supply.

2. It’s non-waterproof, so don’t touch any more liquid like water and oil.

IV. After-sale

1. Guarantee period: One year warranty.